GROUP 23

Mensah Jake Yayra - 7275723

Kyeremateng Oliver- 7274523

⁠Desmond Ndefo - 7276523

⁠Kyei-Sarpong Nana Kesse - 7274423

Ofosu Ebenezer Kwabe - 7278823

Nansii Hamdan Wisinia- 7276323

Dzordzoe Eyram Noamesi- 7277123

Tetteh Nartey – 7276423

Ntiamoah Ernest - 7277623

Reaction Time Tester

Project Documentation

Arduino  •  Tinkercad Simulation

Submitted in partial fulfilment of course project requirements

# 1.  Project Overview

The Reaction Time Tester is a microcontroller-based system designed and implemented by Group 23 using an Arduino Uno and simulated on the Tinkercad platform. The system measures how quickly a person responds to a simultaneous visual and audio stimulus — an LED and a buzzer — and reports the result in milliseconds on a 16x2 LCD screen.

Upon activation, the system introduces a random delay before triggering the stimulus, ensuring the user cannot anticipate the moment of response. Once the stimulus fires, the user presses a push button and the Arduino calculates the elapsed time between stimulus and response. A verdict is then displayed: 'That's fast!' for reaction times under 250 milliseconds, or 'Too slow!' for 250 milliseconds and above. The system loops continuously, allowing repeated testing without any manual reset.

# 2.  Components

The following components were used in the construction and simulation of the project:

| Component | Specification | Qty |
| --- | --- | --- |
| Arduino Uno | ATmega328P microcontroller board | 1 |
| LCD Display | 16x2 character, HD44780 compatible | 1 |
| LED | Any colour, 5mm standard | 1 |
| Push Button | Momentary tactile switch | 1 |
| Buzzer | Passive or active buzzer | 1 |
| Resistor | 220Ω — LED anode and LCD backlight | 2 |
| Potentiometer | 10kΩ — LCD contrast adjustment | 1 |
| Breadboard | Full or half size | 1 |
| Jumper Wires | Male-to-male | As needed |

# 3.  Circuit Wiring

## 3.1  LCD Display (16x2)

The LCD requires six data/control connections to the Arduino, plus power and contrast wiring. All connections are listed below:

| LCD Pin | Label | Connects To |
| --- | --- | --- |
| 1 | VSS | GND |
| 2 | VDD | 5V |
| 3 | V0 (Contrast) | Potentiometer middle pin (wiper) |
| 4 | RS | Arduino Pin 12 |
| 5 | RW | GND |
| 6 | E (Enable) | Arduino Pin 11 |
| 11 | D4 | Arduino Pin 5 |
| 12 | D5 | Arduino Pin 4 |
| 13 | D6 | Arduino Pin 3 |
| 14 | D7 | Arduino Pin 2 |
| 15 | A (Backlight +) | 5V via 220Ω resistor |
| 16 | K (Backlight −) | GND |

## 3.2  Potentiometer (Contrast Control)

The potentiometer adjusts the LCD contrast and does not connect to the Arduino at any point.

- Left outer pin → 5V

- Middle pin (wiper) → LCD V0

- Right outer pin → GND

Turn the potentiometer knob during simulation until the LCD text is clearly visible. Alternatively, connecting V0 directly to GND sets maximum contrast without needing a potentiometer.

## 3.3  LED

- Arduino Pin 9 → 220Ω resistor → LED anode (+)

- LED cathode (−) → GND

## 3.4  Push Button

- One leg → Arduino Pin 7

- Other leg → GND

No external pull-down resistor is required. The firmware configures the pin with INPUT_PULLUP, which enables Arduino's internal pull-up resistor. The pin reads HIGH by default and LOW when the button is pressed.

## 3.5  Buzzer

- Positive leg → Arduino Pin 8

- Negative leg → GND

# 4.  Source Code

## 4.1  Complete Code Listing

#include <LiquidCrystal.h>

// Pin assignments

LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

const int LED    = 9;

const int BUTTON = 7;

const int BUZZER = 8;

void setup() {

pinMode(LED, OUTPUT);

pinMode(BUTTON, INPUT_PULLUP);

pinMode(BUZZER, OUTPUT);

randomSeed(analogRead(A0));   // Seed from analog noise for unpredictability

lcd.begin(16, 2);

}

void loop() {

// Step 1: Wait for user to start a round

lcd.clear();

lcd.print("Press to start!");

while (digitalRead(BUTTON) == HIGH);

delay(200);                          // Debounce

// Step 2: Random delay before stimulus (1 to 4 seconds)

lcd.clear();

lcd.print("Get ready...");

delay(random(1000, 4000));

// Step 3: Fire the stimulus

digitalWrite(LED, HIGH);

tone(BUZZER, 1000, 500);             // 1000Hz tone for 500ms

lcd.clear();

lcd.print("PRESS NOW!");

unsigned long startTime = millis();  // Record stimulus time

// Step 4: Wait for the user to react

while (digitalRead(BUTTON) == HIGH);

// Step 5: Calculate and display result

unsigned long reactionTime = millis() - startTime;

digitalWrite(LED, LOW);

lcd.clear();

lcd.print("Time: ");

lcd.print(reactionTime);

lcd.print(" ms");

lcd.setCursor(0, 1);

if (reactionTime < 250) {

lcd.print("Thats fast!");

} else {

lcd.print("Too slow!");

}

delay(3000);   // Display result for 3 seconds before next round

}

## 4.2  Code Explanation

Libraries and Pin Definitions

The LiquidCrystal library is included to manage communication with the LCD display. The six pins passed to the LiquidCrystal constructor correspond to RS, Enable, D4, D5, D6, and D7 on the LCD. Pin constants are defined at the top of the sketch so they can be changed in one place if the circuit is rewired.

setup()

This function executes once at power-on. It sets the LED and buzzer pins as outputs and the button pin as an input. The INPUT_PULLUP mode activates Arduino's built-in internal resistor, holding the pin HIGH by default without requiring an external component. The randomSeed() call reads electrical noise from the floating analog pin A0, producing a unique seed value each time the board starts so the random delays are never the same sequence twice.

loop() — Step by Step

The entire game logic is placed inside loop(), which Arduino repeats indefinitely. This means the tester resets and runs again automatically after each round with no manual intervention required.

- Step 1 — Idle: The LCD shows 'Press to start!' and execution halts in a while loop until the button is pressed. A 200ms delay follows to debounce the button signal.

- Step 2 — Get Ready: The LCD shows 'Get ready...' and a random delay between 1000 and 4000 milliseconds is applied. This prevents the user from anticipating the stimulus.

- Step 3 — Stimulus: The LED turns on, the buzzer sounds at 1000Hz for 500ms, and the LCD displays 'PRESS NOW!'. The current millisecond count is saved as startTime using the millis() function.

- Step 4 — Wait: Execution pauses in a while loop, doing nothing until the button is pressed.

- Step 5 — Result: The moment the button is pressed, reactionTime is calculated as millis() minus startTime. The LED turns off, and the LCD displays the time in milliseconds on the first row and a verdict on the second row. The result is held for 3 seconds before the loop repeats.

## 4.3  Key Functions Reference

| Function | Description | Where Used |
| --- | --- | --- |
| millis() | Returns ms elapsed since board started | Reaction time calculation |
| random(min, max) | Generates a random integer in range | Pre-stimulus delay |
| randomSeed(val) | Seeds the random number generator | setup() — prevents repeated patterns |
| tone(pin, freq, ms) | Plays a frequency on buzzer for duration | Stimulus activation |
| INPUT_PULLUP | Enables internal pull-up resistor | Button pin configuration |
| lcd.setCursor(col, row) | Moves LCD write position | Printing verdict on row 2 |
| lcd.clear() | Clears all LCD content | Between each display state |

# 5.  System Operation

The following describes the complete flow of one round from start to finish:

- Power on — the system initialises and the LCD displays 'Press to start!'

- The user presses the button to begin the round.

- The LCD displays 'Get ready...' and a random delay between 1 and 4 seconds elapses.

- The LED lights up and the buzzer plays a tone — this is the stimulus.

- The user presses the button as quickly as possible in response.

- The LED and buzzer deactivate. The LCD shows the reaction time in milliseconds on the first row.

- The second row displays 'That's fast!' if the time is under 250ms, or 'Too slow!' otherwise.

- After 3 seconds the display clears and the system returns to step 1 automatically.

# 6.  LCD Messages Reference

The LCD is the primary interface between the system and the user. The following messages appear at different stages of each round:

| Message | Row | Stage | Meaning |
| --- | --- | --- | --- |
| Press to start! | 1 | Idle | System is waiting for the user to begin a round |
| Get ready... | 1 | Pre-stimulus | Random delay in progress — stimulus has not fired yet |
| PRESS NOW! | 1 | Stimulus active | LED and buzzer are on — user should react immediately |
| Time: XXX ms | 1 | Result | Measured reaction time displayed in milliseconds |
| That's fast! | 2 | Result | Reaction time was under 250 milliseconds |
| Too slow! | 2 | Result | Reaction time was 250 milliseconds or above |

# 7.  Challenges and Solutions

The following challenges were encountered during implementation and resolved as described:

| Challenge | Root Cause | Solution |
| --- | --- | --- |
| LCD displayed nothing | V0 contrast pin left unconnected | Connected V0 to potentiometer wiper; alternatively, connect directly to GND for maximum contrast |
| LCD backlight not illuminating | A and K pins not wired | Connected A to 5V via 220Ω resistor and K to GND |
| Button triggering erratically | Floating pin with no defined state | Used INPUT_PULLUP to hold pin HIGH by default; button press pulls it LOW |
| Same random delays every restart | randomSeed() not called | Added randomSeed(analogRead(A0)) in setup() to seed from analog noise |
| Buzzer producing no sound | Buzzer pin not configured as output | Added pinMode(BUZZER, OUTPUT) in setup() |

# 8.  Possible Improvements

The following enhancements could be made to extend the functionality of the project in future iterations:

- Persistent best-time tracker — store and display the fastest reaction time across all rounds in the current session.

- False start detection — if the button is pressed during the 'Get ready...' phase, disqualify the round and display a penalty message.

- Two-player competitive mode — use two buttons and declare the faster respondent the winner.

- Difficulty levels — adjust the 250ms threshold or vary the stimulus duration to make the test easier or harder.

- Serial output — log all reaction times to the Arduino Serial Monitor for data collection and analysis.

- EEPROM storage — save the all-time best score to Arduino's non-volatile memory so it persists across power cycles.

# 9.  References

- Arduino Language Reference — millis(): https://www.arduino.cc/reference/en/language/functions/time/millis/

- Arduino Language Reference — tone(): https://www.arduino.cc/reference/en/language/functions/advanced-io/tone/

- Arduino Language Reference — randomSeed(): https://www.arduino.cc/reference/en/language/functions/random-numbers/randomseed/

- LiquidCrystal Library Documentation: https://www.arduino.cc/en/Reference/LiquidCrystal

- Tinkercad Online Simulation Platform: https://www.tinkercad.com